FAS-G

Gyro Enhanced Inclinometer with Analogue Output

The FAS-G operations manual, orientation, and connection can be found at the following locations, respectively:

http://www.microstrain.com/manuals/FASGUser122.pdf

http://www.microstrain.com/pdf/3025-0003.pdf

http://www.microstrain.com/pdf/3025-0002.pdf

FAS-G combines an angular rate gyro with two orthogonal DC accelerometers, 12 bit A/D converter, multiplexer, microcontroller, and D/A converter to provide an analog voltage linearly proportional to inclination in dynamic and static environments.

Operating over the full 360 degrees of angular motion, FAS-G is versatile by design. The additional digital serial output can be programmed to provide either compensated angles or raw sensor data. The digital output modes and software filter parameters are user programmable. All programmed parameters are stored in nonvolatile memory.

Specifications:

| Range | Single axis(roll) 360 degrees |
| --- | --- |
| A/D Resolution | 12 bits |
| D/A Resolution | 12 bits |
| Dynamic Compensation | Closed loop digital control (0 to 50 Hz) |
| Angle Resolution | Roll: < 0.1 degrees (angle resolution specs. taken at most aggressive setting) |
| Temperature Drift | +/- 0.025%/deg C |
| Nonlinearity | 0.23% full scale (static) |
| Repeatability | 0.10 degrees |
| Accuracy | Roll: +/-1.0 degrees typical |
| Output Data Rate | 150 Hz (digital and analog) |
| Output Modes | single axis: 0-360 deg; angular rate: 300 deg/sec; accelerometer: ax, ay Bits) |
| Output Format | RS-232 serial or analog (0-4.096v) |
| Transmission Rate | 19200 bits/sec |
| Supply Voltage | +5.2 VDC min., +12 VDC max. |
| Supply Current | 30 milliamps |
| Connector | 8 pin mini DIN |
| Operating Temperature | -40 to +50 deg Celsius |
| Enclosure (w/tabs) | 64mm by 90 mm by 25mm; 2.5" by 3.5"' by 1.0" |
| Shock Limit | 1000g (unpowered); 500g (powered) |